RISC-V Architecture

Introduction : This document focuses and summarize the aspects of RISC-V architecture that is required to implement said architecture specially The register fie and instruction set architecture I assume you already have some knowledge regarding RISC-V assembly if you are unfamiliar with this assembly than I recommend using the references in the end of the document.

Register File :

RISC architecture Defines 32 registers specified in the following table:

| Name | Reg number | Objective |
| --- | --- | --- |
| Zero | x0 | Hardwired to ground |
| ra | x1 | Return address |
| Sp | x2 | Stack pointer |
| gp | x3 | Global pointer |
| tp | x4 | Thread pointer |
| t0-2 | x5-7 | Temporary Registers |
| s0/fp | x8 | Saved register/frame pointer |
| S1 | x9 | Saved register |
| a0-1 | x10-11 | Function arg/Return Variables |
| a2-7 | x12-17 | Function arguments |
| s2-11 | x18-27 | Saved registers |
| t3-6 | x28-31 | Temporary registers |

RISC-V ISA (Instruction Set Architecture):

All instructions have a constant width of 32-bit, the instructions forms are divided into four main forms R-type, I-type, S/B-type and U/J type instructions.

R-type instructions : This type of instructions operates on three registers two act as source registers (rs, rt) and one as a destination register rd.

| Funct7 | rs | rt | Funt3 | rd | op |
| --- | --- | --- | --- | --- | --- |
| 7bit | 5bit | 5bit | 3bit | 5bit | 7bit |

I-type instruction : This form of instructions work on two registers and one immediate, one of the registers act as a destination register the other act as a source the second value comes from a 12 bit signed immediate (constant) defined in the instruction itself.

| Imm11:0 | rs | Funt3 | rd | op |
| --- | --- | --- | --- | --- |
| 12bit | 5bit | 3bit | 5bit | 7bit |

Remark that Risc-V operations only operate on 32-operators so the 12-bit signed immediate must be sign extended depending on the MSB, a positive number like +5 will be extended with 0’s as shown 000000000101=>000……00101.

meanwhile -5 =>111111111011(12-bit)=>111…..1011(32-bit).

S/B- type instructions: Store/branch instructions both use two source registers and an immediate, for store instruction one register holds the value to be stored in memory while the other register holds the base address with an offset stored in the 12-bit immediate, For the branch instruction an operation is done the two registers and the branch is taken with an offset stored in the 13-bit immediate encoded with 12-bits this is possible because the first bit is always 0, branches jump between instructions which are hold in 4 bytes (risc-v is byte addressable architecture).

| S | Imm11:5 | rs2 | rs1 | Funct3 | Imm4:0 | op |
| --- | --- | --- | --- | --- | --- | --- |
| B | Imm12,10:5 | rs2 | rs1 | Funct3 | Imm4:1,11 | op |
|  | 7bit | 5bit | 5bit | 3bit | 5bit | 7bit |

The sequence of immediate bits may seem a bit strange but it’s order in such a way to ease the decoding process which will prove useful to the developer.

U/J-type instructions: This form of instructions contain a 20-bit immediate and a destination register, for Upper immediate instruction the 20 MSB’s are loaded into the destination register while the other 12 bits are left unchanged, For Jump instructions  jal (jump and link) instruction the immediate represent an offset relative to the PC NOTE that J-type instructions use a 21-bit immediate encoded with 20 bits just like B-type instructions after the jump the address of the PC (before the offset) is stored into the destination register if no register is specified then it defaults to ra (reg number is x1).

| U | Imm31:12 | rd | op |
| --- | --- | --- | --- |
| J | Imm20,10:1,11,19:12 | rd | op |
|  | 20bit | 5bit | 7bit |

Like B-type J-type immediate bits are scrambled.

The following figure shows how The immediate is encoded in each instruction type.

| Imm11 |  | Imm11:1 | Imm0 | I,S |
| --- | --- | --- | --- | --- |
| Imm12 |  | Imm11:1 | 0 | B |
| Imm31:21 | Imm20:12 | 0 |  | U |
| Imm20 | Imm20:12 | Imm11:1 | 0 | J |
| 11bits | 9bits | 11bits | 1bit |  |

Also the following table compares all instruction types side to side:

| funct7 | rs2 | rs1 | funct3 | rd | op | R |
| --- | --- | --- | --- | --- | --- | --- |
| imm11:0 |  | rs1 | funct3 | rd | op | I |
| imm11:5 | rs2 | rs1 | funct3 | imm4:0 | op | S |
| imm12:,10:5 | rs2 | rs1 | funct3 | imm4:1,11 | op | B |
| imm31:12 |  |  |  | rd | op | U |
| imm20,10:1,11,19:12 |  |  |  | rd | op | J |
| 7bits | 5bits | 5bits | 3bits | 5bits | 7bits |

The following table will introduce RV32I: Base integer instruction set (We will be implementing this instructions) they are part of the standard instruction set extensions can dbe added to support floating numbers, 64 and 128 bit integers and many other instructions but we will stick with only the basic instructions.

| op | Funct3 | Funct7 | Type | Instruction | Description | Operation |
| --- | --- | --- | --- | --- | --- | --- |
| 0000011 (3) | 000 | - | I | lb rd, imm(rs1) | load byte | rd <= SignExt([Address7:0]) |
| 0000011 (3) | 001 | - | I | lh rd, imm(rs1) | load half | rd<= SignExt([Address15:0]) |
| 0000011 (3) | 010 | - | I | lw rd, imm(rs1) | load word | rd<= SignExt([Address31:0]) |
| 0000011 (3) | 100 | - | I | lbu rd, imm(rs1) | load byte unsigned | rd<= ZeroExt([Address7:0]) |
| 0000011 (3) | 101 | - | I | lhu rd, imm(rs1) | load half unsigned | rd<= ZeroExt([Address15:0]) |
| 0010011 (19) | 000 | - | I | addi rd, rs1, imm | add immediate | rd<= rs1 + SignExt(imm) |
| 0010011 (19) | 001 | 0000000 | I | slli rd, rs1, shamt* | shift logical left immediate | rd<= rs1 << shamt |
| 0010011 (19) | 010 | - | I | slti rd, rs1, imm | set less than immediate | rd<= (rs1 < SignExt(imm)) |
| 0010011 (19) | 011 | - | I | xori rd, rs1, imm | Xor immediate | rd<= rs xor SignExt(imm) |
| 0010011 (19) | 100 | - | I | srli rd, rs1, shamt | shift right logical immediate | rd<= rs >> shamt |
| 0010011 (19) | 101 | 0000000 | I | srai rd, rs1, imm | shift right Arithmetic immediate | rd<= rs >>> shamt |
| 0010011 (19) | 110 | 0100000 | I | ori rd, rs1, imm | or immediate | rd<= rs or imm |
| 0010011 (19) | 111 | - | I | andi rd, rs1, imm | and immediate | rd<= rs and imm |
| 0010111 (23) | - | - | U | auipc rd, upimm | add upper immediate to PC | rd<= (upimm & 12’b0) + PC |
| 0100011 (35) | 000 | - | S | sb rs2, imm(rs1) | store byte | [address]7:0 <= rs27:0 |
| 0100011 (35) | 001 | - | S | sh rs2, imm(rs1) | store half | [address]15:0 <= rs215:0 |
| 0100011 (35) | 010 | - | S | sw rs2, imm(rs1) | store word | [address]31:0 <= rs231:0 |
| 0110011 (51) | 000 | 0000000 | R | add rd, rs1,rs2 | add | rd<= rs1 +rs2 |
| 0110011 (51) | 000 | 0100000 | R | sub rd, rs1,rs2 | sub | rd<= rs1 – rs2 |
| 0110011 (51) | 001 | 0000000 | R | sll rd, rs1,rs2 | shift left logical | rd<= rs1<< rs24:0 |
| 0110011 (51) | 010 | 0000000 | R | slt rd, rs1,rs2 | set less than | rd <= (rs1 < rs2) |
| 0110011 (51) | 011 | 0000000 | R | sltu rd, rs1,rs2 | set less then unsinged | rd <= (rs1 < rs2) |
| 0110011 (51) | 100 | 0000000 | R | xor rd, rs1,rs2 | xor | rd <= (rs1 xor rs2) |
| 0110011 (51) | 101 | 0000000 | R | srl rd, rs1,rs2 | shift right logical | rd <= (rs1 >> rs24:0) |
| 0110011 (51) | 101 | 0100000 | R | sra rd, rs1,rs2 | shift right arithmetic | rd <= (rs1 >>> rs24:0) |
| 0110011 (51) | 110 | 0000000 | R | or rd, rs1,rs2 | or | rd <= (rs1 or rs2) |
| 0110011 (51) | 111 | 0000000 | R | and rd, rs1,rs2 | and | rd <= (rs1 and rs2) |
| 0110111 (55) | - | - | U | lui rd, upimm | load upper immediate | rd<= (upimm & 12’b0) |
| 1100011 (99) | 000 | - | B | beq rs1, rs2, label | branch if = | if (rs1 = rs2) then PC=BTA |
| 1100011 (99) | 001 | - | B | bne rs1, rs2, label | branch if ≠ | if (rs1 ≠ rs2) then PC=BTA |
| 1100011 (99) | 100 | - | B | blt rs1, rs2, label | branch if < | if (rs1 < rs2) then PC=BTA |
| 1100011 (99) | 101 | - | B | bge rs1, rs2, label | branch if ≥ | if (rs1 ≥rs2) then PC=BTA |
| 1100011 (99) | 110 | - | B | bltu rs1, rs2, label | branch if < unsigned | if (rs1 < rs2) then PC=BTA |
| 1100011 (99) | 111 | - | B | bgeu rs1, rs2, label | branch if ≥ unsigned | if (rs1 ≥ rs2) then PC=BTA |
| 1100111 (103) | 000 | - | I | jalr rd, rs1, imm | jump and link register | PC<= rs1 + SignExt (imm), rd <= PC + 4 |
| 1101111 (111) | - | - | J | jal rd, label | jump and link | PC<= JTA , rd<= PC + 4 |

In black: Aren’t implemented.

In Orange: Missing control signals from the control unit.

In green: Implemented.

Remarks Concerning the instructions:

Shamt : Shift amount, encoded in the 5 LSB’s of the immediate.

Imm(rs): the target address is a combination of a base address stored in rs and an offset to EX: 253(zero) mean that the  Target address = zero + 253 = 253.

Architectural State:

A processor can be viewed as a huge Finite state machine or multiple FSM’s the architectural state is the current state of the processor defined by the register file Program counter PC and the memory, Depending on the instruction, data held in the registers and the current instruction the processor will generate a different output, Note that if a program was Interrupted by the Operating System for any reason such as exceptions, the os can save the architectural state use the processor to deal with exception and then restore the architectural state and the program will continue like it was never interrupted.

References:

- Digital Design & Computer Architecture RISC-V Edition (Chapter 6 and 7).